Page 1 of 4

Issue Date: 3rd November 2012

MATERIAL SAFETY DATA SHEET

STATEMENT OF HAZARDOUS NATURE

This product is classified as not hazardous.

COMPANY DETAILS

COMPANY:					Tranzmile Pty Ltd.

ADDRESS:	855 Boundary Road

Coopers Plains Qld 4108

TELEPHONE NUMBER:				(07) 3344 4156

Fax:						(07) 3345 5195

EMERGENCY TELEPHONE NUMBER:		0447 470 072  (Technical Department)

IDENTIFICATION

PRODUCT NAME:				LITH-PLEX PMI-2

OTHER NAMES:				None.

MANUFACTURER’S PRODUCT CODE:		5004302

UN NUMBER:					Not applicable.

HAZCHEM CODE:				Not applicable.

DANGEROUS GOODS

CLASS/SUBSIDIARY RISK:			Not applicable.

POISONS SCHEDULE NUMBER:		Not applicable.

USE	High temperature, extreme pressure  grease for use in heavy earth movimg, mining and industrial applications.

PHYSICAL DESCRIPTION/PROPERTIES

APPEARANCE:					Smooth black grease.

VAPOUR PRESSURE(Pa or mmHg at 20C):	Not available.

DENSITY @ 15oC (g/mL):			Typically 0.89.

FLASHPOINT(C):				>200C(ASTM D-93).

FLAMMABILITY LIMITS(%):			Not available.

SOLUBILITY IN WATER(g/L):			Not miscible with water.

OTHER PROPERTIES

BOILING POINT (0C):				> 316 0C.

DROP POINT (0C):				> 260 0C.

VISCOSITY @ 40 0C(cSt):			>Typically 460.

PENETRATION, UNWORKED:			Typically 280.

Page 2 of 4

Product: Lith-Plex PMI-2

MATERIAL SAFETY DATA SHEET

IDENTIFICATION continued

INGREDIENTS

CHEMICAL ENTITY:								CAS No.	PROPORTION

Residual Oils(Petroleum), Solvent Dewaxed						64742-62-7	> 60%

Lithium 12-Hydroxy Stearate thickener						8001-78-3	< 15%

Mixture of Sulphur and Phosphorus Compounds					      n/a		<10%

Zinc Dialkyl Dithiophosphate						 	68649-42-3	<10%

Polyolefin									      n/a		<10%

Proprietary additive system							  		< 10%

Proprietary solids system										< 20%

HEALTH HAZARD INFORMATION

HEALTH EFFECTS

ACUTE		SWALLOWED:			This is not expected to be a means of entry during routine operation.

Ingestion of small quantities should not cause irritation.

EYE:				May cause slight irritation to the eyes.

SKIN:				May be mildly irritating to the skin.  High pressure injection

through the skin, when using apparatus such as grease guns, can be

highly irriating and may cause localised damage.

INHALED:			No data to indicate a toxic inhalation hazard.  Inhalation of vapours 						or mist (generated at elevated temperatures) may cause irritation to 						the nose and throat.

CHRONIC:					Prolonged or repeated skin contact may lead to drying of the skin.

FIRST AID	SWALLOWED:			If a large quantity is ingested seek medical attention.  It may be

necessary to induce vomiting, taking extreme care that the person

does not aspirate into the lungs.  If aspiration occurs, urgent medical

attention should be sought.

EYE:				Wash with copious amounts of water for 15 minutes and seek 							medical advice if irritation develops or persists.

SKIN:				Remove contaminated clothing and wash skin thoroughly with

plenty of soap and water.  High pressure injection through the skin

requires urgent medical attention for possible incision, irrigation

and/or debridement.  Contact with molten material will require

treatment by a physician for burns(Do not remove material).

INHALED:			Remove person to fresh air and seek medical advice. If not 							breathing, apply artificial respiration and seek urgent medical aid.

FIRST AID FACILITIES:		Eye wash fountains and safety showers are recommended.

ADVICE TO DOCTOR:	Treat symptomatically.

Page 3 of 4

Product: Lith-Plex PMI-2

MATERIAL SAFETY DATA SHEET

PRECAUTIONS FOR USE

EXPOSURE STANDARDS:			If the material is subjected to high temperature operations and mists

or vapours are generated the Manufacturer recommends:

Time Weighted Average (TWA):  5 mg/m3  (oil mist).

Short Term Exposure Limit (STEL): 10 mg/m3  (oil mist).

ENGINEERING CONTROLS:			Special ventilation is not normally required. However, in the

operation of certain equipment or at elevated temperatures mists or

vapour may be generated and localised exhaust ventilation should be

provided to maintain airborne concentration levels below the

manufacturer recommended exposure standard.

PERSONAL PROTECTION

RESPIRATOR TYPE (AS1716):			During routine operation a respirator is not required.  However, if 						mists or vapours are generated, an approved organic vapour/							particulate respirator is required.

GLOVE TYPE:					PVC, Neoprene or Nitrile gloves are recommended.

EYE PROTECTION:				Safety glasses or goggles are recommended to avoid eye contact.  If

the material is used at elevated temperatures or under pressure a full

face shield should be worn.

CLOTHING:					During normal operating procedures, long sleeved clothing is 							recommended to provide skin protection.  Soiled clothing should be 						washed with detergent prior to re-use.

FLAMMABILITY

FIRE HAZARDS:				Not flammable under conditions of use.  Is a combustible solid that 						will burn if preheated.

SOURCES OF IGNITION ADVICE:		Not applicable.

HAZCHEM CODE:				Not applicable.

SAFE HANDLING INFORMATION

STORAGE AND TRANSPORT:			Classified as a combustible solid.  Store in a well ventilated area

away from ignition sources, oxidising agents, foodstuffs and

clothing. Keep containers closed when not in use.  Do not store in

plastic containers unless approved for this application.

SPILLS AND DISPOSAL:			Spills are easily contained due to the nature of the product.  Caution:

The product may be slippery.  The product should be shovelled into

a metal drum and treated as a solid waste.  Follow state or local

regulations for the disposal of the waste. Clean area with soap and

water. Do not allow product to enter drains, sewers or water courses

– inform local authorities if this occurs.

Page 4 of 4

Product: Lith-Plex PMI-2

MATERIAL SAFETY DATA SHEET

SAFE HANDLING INFORMATION continued

FIRE/EXPLOSION HAZARD

HAZARDS OF USE/STORAGE:			This product is combustible if preheated.

LIST OF DANGEROUS DECOMPOSITION

OR COMBUSTION PRODUCTS:	Combustion produces oxides of carbon, nitrogen, sulphur, phosporus, molybdenum and zinc.

FIRE-FIGHTING RECOMMENDATIONS:		Keep storage tanks, pipelines, containers,  fire exposed surfaces, etc.

cool with water spray.

TYPES OF EXTINGUISHER/

FIRE-FIGHTING AGENTS:			Use foam, carbon dioxide or dry chemical.

PRECAUTION:					Water may cause splattering.

PROTECTIVE CLOTHING:			Self-contained breathing apparatus is recommended during a fire.

REACTIVITY:					May react with strong oxidising agents.

OTHER INFORMATION

TOXICOLOGICAL DATA:

ORAL TOXICITY(RATS): Based upon testing of similar products and/or components, this material is considered to be relatively non-toxic with a LD50: > 2000 mg/Kg.

DERMAL TOXICITY(RABBITS): Based upon testing of similar products and/or components, this material is considered to be relatively non-toxic with a LD50: > 2000 mg/Kg.

EYE IRRITATION(RABBITS): Based upon testing of similar products and/or components, this material is considered to be relatively non-irritating with a Draize Score: Greater than 6, but less than 15.

SKIN IRRITATION(RABBITS): Based upon testing of similar products and/or components, this material is considered to be relatively non-irritating with a Primary Irritation Index: Greater than 0.5, but less than 3.

SUBCHRONIC TOXICITY(RATS): Based upon testing of similar products and/or components, this material is considered to show no adverse effects by dermal application to rats.

REPRODUCTIVE TOXICITY(RATS): Based upon testing of similar products and/or components, this material is considered to show no adverse effects in either the mothers or their offspring by dermal exposure of pregnant rats.

CHRONIC TOXICITY(MICE): Based upon testing of similar products and/or components, this material is considered to show no evidence of carcinogenic effects by chronic mouse skin painting studies.

SENSITISATION(GUINEA PIGS): Based upon testing of similar products and/or components, this material is considered to show no evidence of skin sensitisation to guinea pigs during studies.

CONTACT POINT

TITLE:						Technical Department Supervisor

TELEPHONE NUMBER:				(07) 3344 4156

MOBILE:					 0447 470 072

All information contained in this Material Safety Data Sheet and the health, safety and environmental information are considered to be accurate to the best of our knowledge as of the issue date specified above. However, no warranty or  representation, expressed or implied, is made as to the accuracy or completeness of the data and information contained in this data sheet.

Health and safety precautions and environmental advice noted in this data sheet may not be accurate for all individuals and/or situations. It is the user’s obligation to evaluate and use this product safely and to comply with all applicable laws and regulations. The Company accepts no responsibility for any injury, loss or damage, resulting from abnormal use of the material, from any failure to adhere to recommendations, or from any hazards inherent in the nature of the material.